产品单页介绍 — F1220

文档编号：	SPI-F1220-REV02-2021

产品型号：	F1220

产品名称：	手持RTN高斯计

保修服务：	1年有限保修服务（不包含霍尔探头）

校准服务：	1次免费校准

计量服务：	非强制计量产品、付费开具计量证书

使用标准：	EIA-232-E、USB 1.1 Standard、IEC 825-1

产品特点：	 直流测量		 测量范围 30kG、6kG、600G DC

 显示分辨率3 3/4、3 6/7位，测量分辨率10G、1G、0.1G

 准确度：（2%读数+0.05%量程）@ 25C10C

 读数速率：10读数/s

 交流测量		 测量范围 3.6kG、1.2kG、120G RMS

 显示分辨率2 3/4、3 1/2位，测量分辨率10G、1G、0.1G

 准确度：3%读数@ 25C10C（30Hz—10kHz、正弦波）

 读数速率：5读数/s

 霍尔探头		 专有RTN零点稳定和降噪技术

 通用玻璃纤维封装免校准数字化横向霍尔探头

 通用黄铜封装免校准数字化轴向霍尔探头

 多点线性校正

 温度系数	(0.05%读数+0.02G)/C @ 25C10C

 探头自动校零功能

 自动控制		 标准配备内部USB—RS-232C转换接口和易用的接口特性

 提供完整命令集

 交互能力		 亮度可调的白色背光FSTN LCD显示器

 与显示器配合的7键交互型功能键盘操作

 数学计算		 最大值保持和相对值测量

  G、T、A/m单位变换

 供电和充电	 内部1850mAh锂聚合物电池供电

 使用标准USB接口（500mA）充电

 内置锂电池充电管理器和LED指示器

 充电接口同时作为通讯接口

 标准规格		 提供用户手册

 手持仪器（配置手提箱）

 外形尺寸1739142mm

标准配置：	 仪器		F1220				手持RNT高斯计						1台

 探头		F12201				数字化玻璃纤维封装横向霍尔探头		1只

 附件		CAB-USB-AB-0.5		USB标准A-B电缆0.5m				1条

 光盘		CD-F1220	 		产品信息光盘						1片

内含：				MAN-F1220		用户手册电子文档

DRV-USB232		USB接口驱动程序

 手提箱	CASE37D-1			含填充物的手提箱					1只

可选配置：	探头		F12202				数字化黄铜封装轴向霍尔探头

SINGLE PAGE INTRODUCTION - F1220

Document Number：	SPI-F1220-REV02-2021

Model:	F1220

Name:	Hand Held RTN Gaussmeter

Warranty:		Limited warranty for a period of 1 year from delivery（except hall probes）

Calibration:	Free for the first time calibration

Measuration: 	Without force measuration product, payment for Certification of China

Standard:		Follows EIA-232-E, USB 1.1 Standard and IEC 825-1

Specification:	 DC Measurement	 Range of 30kG, 6kG, 600G DC

 Resolution of 3 3/4, 3 6/7，or 10G, 1G, 0.1G

 Accuracy of (2% reading+0.05% range) @ 25C10C

 10 reading/s

 AC Measurement	 Range of 3.6kG, 1.2kG, 120G RMS

 Resolution of 2 3/4, 3 1/2，or 10G, 1G, 0.1G

 Accuracy of 3% reading @ 25C10C for sine wave

 5 reading/s

 Probe				 RTN technology for high zero stability and noise rejection

 Fiberglass sealed digital transverse hall probes

 Brass sealed digital axial hall probes

 Linearity compensation

 TR of 	(0.05% reading+0.02G)/ C @ 25C10C

 Probe auto zero function

 Automation		 Internal USB-RS232C converter

 Full commands provided

 Human Interface	 FSTN LCD display with adjustable white back lighting

 7 functional keyboard operation

 Math Functions		 Maximum hold and relative readings

 G, T, A/m unit conversion

 Battery			 Internal 1850mAh Li-Poly Battery

 Charging with USB Interface of no less than 500mA

 Internal charging management and indicators

 Shared with communication interface

 Standard Spec.		 User’s manual and device driver

 Hand held instrument with suitcase

 Dimension of HWD =1739142mm

Standard Configuration:

 Instruments		F1220			Hand Held RTN Gaussmeter		1

 Probes			F12201 			Fiberglass sealed transverse probe	1

 Accessories		CAB-USB-AB-0.5	Std. USB A-B cable of 0.5m long	1

 CDs				CD-F1220		Product information CD			1

which contains:	MAN-F1220 user’s manual e-documents

DRV-USB232 interface driver

 Suitcase			CASE37D-1		Suitcase with filling material		1

Options:		 Probes			F12202			Brass sealed axial probe