产品单页介绍 — F2031

文档编号：	SPI-F2031-REV03-2024

产品型号：	F2031

产品名称：	程控功率电流源

关键词：	5A/60V/300W、强电感、高分辨率、电流渐变、1—12

程控、负载保护

保修服务：	1年有限保修服务

校准服务：	1次免费校准

计量服务：	非强制计量产品、付费开具计量证书

使用标准：	GB-1002、EIA-232-E、USB 1.1 Standard、IEC 825-1

产品特点：	 输出电流		 输出范围：-5A—+5A DC，准双极性

 步进分辨率0.05mA（1/100000 F.S.）

 准确度：（0.1%设置值+1mA）

 线性度：优于0.01%

 温度系数：优于5ppm/C

 短期稳定度：优于30ppm/h

 噪声/纹波：1mA RMS典型值

 直流输出阻抗：>2.5M

 渐进速率：0.01A/s—2A/s

 浮动输出，I、III象限，内部继电器换向

 输出电压		 有效输出范围：60V DC

 开路输出电压：>80V DC

 输出功率		 最大有效输出功率：300W

 适用负载		 最小负载直流电阻1（输出电流5A）

 最大负载直流电阻12（输出电流5A、不触发钳位指示）

 适应强电感性负载

 自动控制		 标准配备RS-232C接口和易用的接口特性

 标准配备内部USB—RS-232C转换接口

 包含极头退磁扫描在内的4种模式电流自动扫描功能

 负载保护输入

 同步触发输出功能

 交互能力		 白色背光FSTN LCD显示器

 必备的LED状态指示器

 与显示器和指示器配合的34键交互型功能键盘操作

 标准规格		 提供用户手册和设备驱动程序

 3U台式仪器，交流220V/50Hz供电

 散热方式：强制风冷

 外形尺寸WHD=247132472mm

标准配置：	 仪器		F2031				程控功率电流源						1台

 附件		CAB-AC250-052 		3芯交流250V/5A GB1002电源线2m	1条

CAB-232-DCE-3		RS-232C标准直通电缆3m				1条

CAB-USB-AB-2		USB标准A-B电缆1.8m				1条

FUSE5C-25040		250V/4A陶瓷保险管，520mm			2只

 光盘		CD-F2031	 		产品信息光盘						1片

内含：				MAN-F2031		用户手册电子文档

DRV-USB232		USB接口驱动程序

Single Page Introduction – F2031

Document Number：	SPI-F2031-REV03-2024

Model:	F2031

Name:	Programmable Power Current Source

Key words:	5A/60V/300W, strong inductance load, high resolution, current ramp, 1-12

Programmable, load protection

Warranty:		Limited warranty for a period of 1 year from delivery

Calibration:	Free for the first time calibration

Measuration: 	Without force measuration product, payment for Certification of China

Standard:		Follows GB-1002, EIA-232-E, USB 1.1 Standard and IEC 825-1

Specification:	 Output Current		 Range of –5A to +5A DC, quasi-bipolar

 0.05mA step resolution, (1/100, 000 F.S.)

 Accuracy of  (0.1% settings+1mA)

 Linearity of better than 0.01%

 Temperature coefficient of better than 5ppm/C

 Short term stability of better than 30ppm/h

 Noise and ripple of 1mA RMS typical

 DC output resistance >2.5M

 Ramp rate from 0.01A/s to 2A/s

 Floating output at I, III region, with internal relay switching

 Output Voltage		 Efficient range of 60V DC

 Open circuit output voltage of higher than 80V

 Output Power		 Efficient power output of 300W max.

 Load Adaptiviey	 Minimum of 1 @ 5A

 Maximum of 12 @ 5A with no compliance occurs

 Capability of driving strong inductance load

 Automation		 RS-232C interface as standard configuration

 Internal USB-RS232 converter

 Current sweep with 4 modes, including pole degauss

 Load protection input interface

 Synchronous triggering output with optional delay time

 Human Interface	 FSTN LCD display with white back lighting and LED indicators

 34 functional keyboard operation

 Standard Spec.		 User’s manual and device drivers

 3U instrument, AC 220V/50Hz supply

 Forced air cooling

 Dimension of WHD=247132472mm

Standard Configuration:

 Instruments		F2031				Programmable Power CS			1

 Accessories		CAB-AC250-052		GB1002 AC 250V/5A 3-conductor	1

power cord of 2m long

CAB-232-DCE-3		Std. RS-232C straight 	through		1

cable of 3m long

CAB-USB-AB-2		Std. USB A-B cable of 1.8m long	1

FUSE5C-25040		250V/4A Ceramic fuse, 520mm	2

 CDs				CD-F2031			Product information CD			1

which contains:		MAN-F2031 user’s manual e-documents

DRV-USB232 interface driver