产品单页介绍 — F2071

文档编号：	SPI-F2071-REV01-2021

产品型号：	F2071

产品名称：	程控功率电流源

关键词：	33A/120V/3.96kW、强电感、高分辨率、电流渐变、2.5—3.6

程控、风冷

保修服务：	1年有限保修服务

校准服务：	1次免费校准

计量服务：	非强制计量产品、付费开具计量证书

使用标准：	ICE60364（GB16895）、EIA-232-E、USB 1.1 Standard、IEC 825-1

产品特点：	 输出电流		 输出范围：-33A—+33 A DC，准双极性

 步进分辨率0.1mA（1/330000 F.S.）

 准确度：（0.2%设置值+6mA）

 线性度：优于0.01%

 温度系数：优于20ppm/C

 短期稳定度：优于100ppm/h

 噪声/纹波：6mA RMS典型值

 直流输出阻抗：>500k

 渐进速率：0.01A/s—5A/s

 浮动输出，I、III象限，内部继电器换向

 输出电压		 有效输出范围：-120—120V DC

 开路输出电压：>145V DC

 输出功率		 最大有效输出功率：3.96kW

 适用负载		 最小负载直流电阻2.5（输出电流0-33A、不触发内部保护）

 最大负载直流电阻3.6（输出电流33A、不触发钳位指示）

 适应强电感性负载

 自动控制		 标准配备RS-232C接口和易用的接口特性

 标准配备内部USB—RS-232C转换接口

 可设置的3种模式电流自动扫描功能

 同步触发输出功能

 交互能力		 白色背光FSTN LCD显示器

 必备的LED状态指示器

 与显示器和指示器配合的34键交互型功能键盘操作

 标准规格		 提供用户手册和设备驱动程序

 标准19英寸机柜仪器，交流380V/50Hz TN-S供电

 散热方式：强制风冷

 外形尺寸WHD=6001300800mm

标准配置：	 仪器		F2071				程控功率电流源						1台

 附件		CAB-AC450-0202 	5芯交流450V/20A电源线2m			1条

CAB-DC050-2			3芯直流50A输出电缆2m				1条

CAB-232-DCE-3		RS-232C标准直通电缆3m				1条

CAB-USB-AFM-2		USB-A延长电缆1.8m					1条

 光盘		CD-F2071	 		产品信息光盘						1片

内含：				MAN-F2071		用户手册电子文档

DRV-USB232		USB接口驱动程序

Single Page Introduction – F2071

Document Number：	SPI-F2071-REV01-2021

Model:	F2071

Name:	Programmable Power Current Source

Key words:	33A/120V/3.96kW, strong inductance load, high resolution, current ramp, 2.5-3.6

Programmable, forced air cooling

Warranty:		Limited warranty for a period of 1 year from delivery

Calibration:	Free for the first time calibration

Measuration: 	Without force measuration product, payment for Certification of China

Standard:		Follows ICE60364 (GB16895), EIA-232-E, USB 1.1 Standard and IEC 825-1

Specification:	 Output Current		 Range of –33A to +33A DC, quasi-bipolar

 0.1mA step resolution, (1/330, 000 F.S.)

 Accuracy of  (0.2% settings+6mA)

 Linearity of better than 0.01%

 Temperature coefficient of better than 20ppm/C

 Short term stability of better than 100ppm/h

 Noise and ripple of 6mA RMS typical

 DC output resistance >500k

 Ramp rate from 0.01A/s to 5A/s

 Floating output at I, III region, with internal relay switching

 Output Voltage		 Efficient range of -120 to 120V DC

 Open circuit output voltage of higher than 145V

 Output Power		 Efficient power output of 3.96kW max.

 Load Adaptiviey	 Minimum of 2.5 @ 0-33A with no internal protection

 Maximum of 3.6 @ 33A with no compliance occurs

 Capability of driving strong inductance load

 Automation		 RS-232C interface as standard configuration

 Internal USB-RS232 converter

 Configurable automatic current sweep with 3 modes

 Load protection input interface

 Synchronous triggering output with optional delay time

 Human Interface	 FSTN LCD display with white back lighting and LED indicators

 34 functional keyboard operation

 Standard Spec.		 User’s manual and device drivers

 Forced air cooling

 Standard 19” cabinet instrument, AC 380V/50Hz TN-S supply

 Dimension of WHD=6001300800mm

Standard Configuration:

 Instruments		F2071				Programmable Power CS			1

 Accessories		CAB-AC450-0202		AC 450V/20A 5-conductor power	1

cord of 2m long

CAB-DC050-2			DC 50A 3-conductor output cable	1

of 2m long

CAB-232-DCE-3		Std. RS-232C straight 	through		1

cable of 3m long

CAB-USB-AFM-2		Std. USB-A extension cable of 		1

1.8m long

 CDs				CD-F2071			Product information CD			1

which contains:		MAN-F2071 user’s manual e-documents

DRV-USB232 interface driver